| PORTAMATRICULAS ADAPTABLE A KAWASAKI ZX6R-09’-14’ LICENSE SUPPORT ADAPTABLE TO KAWASAKI ZX6R-09’-14’ REF:4927N INSTRUCCIONES DE MONTAJE MOUNTING INSTRUCTIONS - Desmontar el soporte original. Dismount the original support. - Montar el soporte suministrado(4), tal y como indica el croquis adjunto, colocando las arandelas(2) y los casquillos(3) entre el soporte y el paso de rueda. Se usan para su fijación los tornillos M6x25(1) junto con las arandelas M6(2) y las tuercas de M6(5) . Se suministran 3 bridas de plástico para fijar los cables de los intermitentes en las ranuras del soporte. Mount the support supplied (4), as the figure below shows, placing the washers (2) and cylinders (3) in between the support and undertail. To tight it, use the M6x25 (1) fasteners jointly with the M6 washers (2) and M6 nuts (5). It is supplied three plastic ziptides to tight the turn signals’ cables in the slots of the support. Material sumistrado para el montaje en la motocicleta: Material supplied: POSICIÓN; DENOMINACIÓN; CANTIDAD. 1; Tornillo M6x25 DIN1001 c/negro; # Black M6x25 screws DIN1001; 4. 2; Arandelas M6 DIN9021; M6 washers DIN9021; 8. 3; Casquillos aluminio; Aluminium cylinders; 4. 4; Soporte portamatriculas; License support; 1. 5; Tuercas M6 DIN985; M6 nuts DIN985; 4. 6; Bridas de plástico; Ziptides; 3. 7; Luz portamatriculas; License support light; 1. 8; Catadrioptico; Reflector; 1. 9; Desplazador catadrioptico; Support for reflector; 1. 10; Tornillos M5x10 ISO7380; M5x10 ISO7380; 2. 11; Tuercas M5 DIN6923; Nut M5 DIN6923; 2. | PORTAMATRICULAS ADAPTABLE A KAWASAKI ZX6R-09’-14’ LICENSE SUPPORT ADAPTABLE TO KAWASAKI ZX6R-09’-14’ REF:4927N INSTRUCCIONES DE MONTAJE MOUNTING INSTRUCTIONS - Desmontar el soporte original. Dismount the original support. - Montar el soporte suministrado(4), tal y como indica el croquis adjunto, colocando las arandelas(2) y los casquillos(3) entre el soporte y el paso de rueda. Se usan para su fijación los tornillos M6x25(1) junto con las arandelas M6(2) y las tuercas de M6(5) . Se suministran 3 bridas de plástico para fijar los cables de los intermitentes en las ranuras del soporte. Mount the support supplied (4), as the figure below shows, placing the washers (2) and cylinders (3) in between the support and undertail. To tight it, use the M6x25 (1) fasteners jointly with the M6 washers (2) and M6 nuts (5). It is supplied three plastic ziptides to tight the turn signals’ cables in the slots of the support. Material sumistrado para el montaje en la motocicleta: Material supplied: POSICIÓN; DENOMINACIÓN; CANTIDAD. 1; Tornillo M6x25 DIN1001 c/negro; # Black M6x25 screws DIN1001; 4. 2; Arandelas M6 DIN9021; M6 washers DIN9021; 8. 3; Casquillos aluminio; Aluminium cylinders; 4. 4; Soporte portamatriculas; License support; 1. 5; Tuercas M6 DIN985; M6 nuts DIN985; 4. 6; Bridas de plástico; Ziptides; 3. 7; Luz portamatriculas; License support light; 1. 8; Catadrioptico; Reflector; 1. 9; Desplazador catadrioptico; Support for reflector; 1. 10; Tornillos M5x10 ISO7380; M5x10 ISO7380; 2. 11; Tuercas M5 DIN6923; Nut M5 DIN6923; 2. |
| --- | --- |