Between 2009 and 2026, Harley-Davidson Touring models use 5 distinct OEM front brake rotor fitments.

Fitment on modern Harley Touring bikes depends heavily on both the rotor diameter and the wheel mounting configuration (traditional hub-mount vs. spoke-mount).

The 5 Touring Front Rotor Fitments

| Fitment Type | Rotor Diameter | Mounting Pattern / Type | Common Models & Years |
| --- | --- | --- | --- |
| 1. Standard 5-Bolt Hub-Mount | 11.8" (300 mm) | 5-bolt pattern on a 3.25" bolt circle (2.22" center bore) | 2009–2023 Standard Touring models (Electra Glide, Road King, Street Glide/Road Glide with cast/laced wheels, Talon wheels); carried over on select models like Road King Special. |
| 2. Enforcer / Prodigy Spoke-Mount | 11.8" (300 mm) | 5-bolt perimeter/spoke-mount pattern (larger center opening, bolts to wheel spokes) | 2014–2023 Street Glide (FLHX) & Road Glide (FLTRX) equipped from the factory with Enforcer, Impeller, or Prodigy wheels. |
| 3. 7-Bolt Agitator Mount | 11.8" (300 mm) | Proprietary 7-bolt hub/spoke pattern | 2010–2023 Select CVO models and factory accessory wheels (Agitator, Chisel, and Slicer wheels). |
| 4. Standard Spoke-Mount (Rushmore Refresh) | 12.6" (320 mm) | 5-bolt spoke-mounted floating rotor | 2024–2026 Standard Touring redesigns (all-new 2024+ Street Glide & Road Glide) and 2023.5 CVO Street Glide / Road Glide. |
| 5. Hub-Mount (CVO / ST Radial) | 12.6" (320 mm) | 5-bolt hub-mounted pattern (similar hub PCD, enlarged 320 mm diameter) | 2023–2026 CVO Street Glide, CVO Road Glide, and CVO Road Glide ST models with Brembo radial calipers. |

Practical Fitment Rules to Keep in Mind

- Rotor Diameter: Touring bikes strictly used 11.8" (300 mm) front discs from 2008 through 2023. Starting with the major redesign in late 2023/2024, the platform upgraded to larger 12.6" (320 mm) rotors paired with updated Brembo caliper geometry.

- Wheel Swaps (Hub vs. Spoke Mount): Between 2014 and 2023, wheels like the Enforcer or Prodigy bolted the rotor directly into the wheel spokes rather than the center hub. If swapping an Enforcer wheel onto a 2009–2013 bike, you must use the Enforcer-specific spoke-mounted rotors (or an aftermarket hub conversion adapter).